| # Safe Work Instruction – Electrical Safety |  |  |  |  |
| --- | --- | --- | --- | --- |
| Note: This Safe Work Instruction provides guidance information only and may not necessarily cover all possible hazards and should be used in conjunction with other references. |  |  |  |  |
| - When using electrical equipment, always check that the power cord has a current electrical test tag. Do not proceed if the next test date has expired or if there is no tag attached |  |  |  |  |
| - Always access power through an outlet protected by an RCD or portable RCD |  |  |  |  |
| - Always give the electrical equipment a thorough inspection prior to use. Do not proceed if faults or damage is detected |  |  |  |  |
| - Remain alert for cords that become worn or damaged. Remember they are sometimes dragged around on concrete or the ground and over sharp edges |  |  |  |  |
| - Regularly inspect switches on electrical equipment. Tag equipment out of use if faults are found |  |  |  |  |
| - If a safety switch/RCD trips during operation of any electrical equipment, always determine the cause and have it rectified prior to continuing work |  |  |  |  |
| - Do not use electrical equipment where it may come into contact with water |  |  |  |  |
| - If using power equipment outside, ensure leads and plugs cannot come into contact with water |  |  |  |  |
| - Where possible use battery or cordless power tools rather than mains powered tools |  |  |  |  |
| - Avoid having electrical cords across walkways, particularly if sharing the worksite with others |  |  |  |  |
| - Remain aware of the location of power cords when using electrical equipment to avoid entanglement and/or trips |  |  |  |  |
| - Keep power cords and leads out of the immediate work space to prevent them being damaged eg. cut, burnt. |  |  |  |  |
| - When unplugging cords from power outlets always turn off the power point first and grip the plug, NOT the cord |  |  |  |  |
| - Do Not use double adaptors as they can become partially unplugged and expose live conductors |  |  |  |  |
| - Do Not plug one power board into another power board as it can lead to overloading the circuit and cause a fire |  |  |  |  |
| - Always ensure your hands are dry whenever handling electrical equipment including switching off power points |  |  |  |  |
| I have read and understand all the above information and agree to abide by all instructions |  |  |  |
| Name | Signature |  | Date |