| # Safe Work Instruction – Gas Bottle Safety |  |  |  |  |
| --- | --- | --- | --- | --- |
| Note: This Safe Work Instruction provides guidance information only and may not necessarily cover all possible hazards and should be used in conjunction with other references. |  |  |  |  |
| ## Handling |  |  |  |  |
| - Never drop bottles from vehicles, trailers, platforms etc. Always have sufficient personnel on hand to safely lift bottles from delivery vehicles |  |  |  |  |
| - Do not allow cylinders to strike each other or any other surface violently |  |  |  |  |
| - Avoid lifting cylinders by the cap or valve |  |  |  |  |
| - Never drag, slide or roll a cylinder. Use a gas cylinder trolley to prevent cylinder falling and to reduce lifting. |  |  |  |  |
| - Always transport bottles securely to prevent movement or cylinders falling. |  |  |  |  |
| - Do not expose cylinders to naked flames, sparks or excessive heat |  |  |  |  |
| - Treat all gas bottles as being full so that empty cylinders are not damaged or stored incorrectly. ‘Empty’ cylinders may still be pressurized and can explode. |  |  |  |  |
| ## Storage |  |  |  |  |
| - Do not store or transport bottles in the horizontal position |  |  |  |  |
| - Ensure bottles are positioned securely. Never leave bottles unsecured with any possibility of falling. (particularly those without a collar) |  |  |  |  |
| - Do not change or store bottles in the presence of naked flames or in the proximity of other ignition sources |  |  |  |  |
| - Store cylinders in a dry area to prevent corrosion |  |  |  |  |
| - Always replace safety caps/plugs when the bottle is empty or not in use |  |  |  |  |
| - No flammable liquids, combustible, corrosive or oxidising material should be stored near a gas storage area. Compressed oxygen must be kept 10m away from all flammable gases |  |  |  |  |
| ## Gas Bottle Use |  |  |  |  |
| - Ask your supplier to set up cylinders and check their operation if you are not experienced to do so |  |  |  |  |
| - Check hoses regularly. Ensure hoses are properly secured with hose clips. If hoses are damaged or showing signs of wear, replace them |  |  |  |  |
| - Ensure you always use the correct valves, hoses and fittings that are designed for the application. If in doubt, contact the manufacturer or your supplier |  |  |  |  |
| - Ensure all connections and hoses are firmly tightened to prevent leaks. |  |  |  |  |
| - Never tamper with or attempt to repair or alter cylinders, regulators or any pressure relief devices. Return cylinders to your gas supplier for all repairs |  |  |  |  |
| - Never use tools to operate bottle valves |  |  |  |  |
| - Make sure the cylinder is equipped with the correct regulator. Always use the regulator designed for the gas in use and that no grease or oil is used on regulator or cylinder valves as these substances may cause a dangerous reaction within the cylinder |  |  |  |  |
| - Prior to removing a regulator from a cylinder, close the cylinder valve and release all pressure from the regulator |  |  |  |  |
| - Always position the cylinder so that the valve at the top is easily accessible at all times |  |  |  |  |
| - The valve should never be left open when equipment is not in use, even when empty; air and moisture may diffuse through an open valve, causing contamination and corrosion within the cylinder |  |  |  |  |
| - Never smoke during the handling of any gas bottle |  |  |  |  |
| I have read and understand all the above information and agree to abide by all instructions |  |  |  |
| Name | Signature |  | Date |