| # Safe Work Instruction – Power Tools |  |  |  |  |
| --- | --- | --- | --- | --- |
| Note: This Safe Work Instruction provides guidance information only and may not necessarily cover all possible hazards and should be used in conjunction with other references. |  |  |  |  |
| ## Preparation |  |  |  |  |
| - Inspect the tool for frayed cords, loose or broken switches, and other obvious problems. Tools that fail this inspection must not be used |  |  |  |  |
| - Check that any guarding is securely in place and functional |  |  |  |  |
| - When necessary, wear appropriate gloves, footwear, dust mask and eye and ear protection while using tool. |  |  |  |  |
| - Do not wear loose clothing or jewellery that may become entangled when operating tools. Tie long hair back |  |  |  |  |
| - Where possible, use battery operated tools in wet areas |  |  |  |  |
| If applicable, ensure you are familiar with the location of the on/off and emergency stop buttons and that they are working correctly |  |  |  |  |
| Work areas should have adequate lighting and be free of clutter |  |  |  |  |
| Ensure observers remain a safe distance away from the work area |  |  |  |  |
| Always stand firmly on the floor/ground and avoid any awkward postures that may cause over balancing |  |  |  |  |
| Make sure immediate work area is clear of other tools, off cut materials or shavings that have the potential to be slipping and tripping hazards |  |  |  |  |
| ## Operating Power Tools |  |  |  |  |
| Avoid accidental starting. Do not hold a finger on the switch button while carrying a plugged-in tool |  |  |  |  |
| Ensure power cords are well clear of the main works area to prevent accidental contact during work process |  |  |  |  |
| Allow the tool to obtain maximum speed before beginning task |  |  |  |  |
| Do not place any body parts, particularly your hands near any moving parts of the tool during operation |  |  |  |  |
| Never leave the power tool running unattended |  |  |  |  |
| Never use power tools around flammable liquids and gases |  |  |  |  |
| Always remain aware of any unusual movements, unusual sounds or other changes during operation. Cease operation if deemed to be faulty or hazardous |  |  |  |  |
| Ensure power tools are sufficiently cooled prior to any maintenance |  |  |  |  |
| Disconnect power tools from the power outlet while servicing or storing |  |  |  |  |
| Do not wrap the cord around the tool for storage to avoid damaging the lead |  |  |  |  |
| Store tools in a dry place |  |  |  |  |
| I have read and understand all the above information and agree to abide by all instructions |  |  |  |
| Name | Signature |  | Date |